Decarbonizing Maritime Shipping: A pathway towards Net-Zero by 2050

By Matthew F. Kurniawan

Introduction

Decarbonizing the maritime shipping industry requires a comprehensive approach, combining technical framework, alternative fuels, and improving current operations and significant infrastructure upgrades at ports, aiming for a net-zero emissions by 2050. The international shipping industry, responsible for about two to three percent of global greenhouse gas (GHG) emissions, requires a significant shift from heavy fuel oils to alternative fuels. The International Marine Organization (IMO) has taken significant steps to address this by adopting the carbon neutrality (Net-Zero) Framework, approved in principle in April 2025 and currently under negotiations for adoption in late 2026. This regulatory framework will be a legally binding element of Annex VI of the MARPOL Convention which will, apply to ships of more than 5,000 gross tonnage. This framework serves as the primary mechanism for driving the energy transition, aiming to phase out fossil fuels and accelerate the adoption of zero or near-zero emission technologies. Alternative fuels include green ammonia, green methanol, hydrogen, and advanced biofuels, which are currently under exploration to achieve long-term sustainability goals. However, significant obstacles exist in the decarbonization of maritime shipping which includes the slow adoption of zero emission fuels, high infrastructure costs, complicated fleet renewal, and regulatory challenges (particularly carbon pricing), which creates significant investment hesitation among shipowners.

The IMO Net-Zero Framework: Technical and Structural Components

The Net-Zero framework applies to ships of 5,000 gross tonnage (GT) and above, covering about 85% of global shipping emissions. Its fundamental principle is based a global fuel standard, which mandates a progressive reduction in the Greenhouse Gas intensity on a lifecycle basis. This standard ensures that emissions from the production, transportation, and the use of fuel are considered, not just those generated during ship operations. To meet these targets, this framework dictates a gradual decrease in allowable emission intensity, becoming progressively stricter through 2035 and beyond, with a 2040 checkpoint targeting a 65% reduction from 2008 levels. Additionally, this NZF establishes a credit trading scheme (emissions trading), allowing over-compliant vessels (those exceeding compliance requirements) to sell excess credits to ships that do not meet their intensity targets. This results in a market-driven approach to reducing GHG emissions, focusing on technology adoption of alternative fuels, rather than prescribing specific fuel types.

Economic Measures: GHG Pricing and the Net-Zero Fund

A fundamental pillar of the IMO’s strategy is the implementation of a global economic measure, designed to bridge the price gap between conventional fossil fuels and more expensive, cleaner alternatives. This pricing mechanism requires vessels that do not meet the GHG fuel intensity targets to buy Remedial Units (RUs). These act as penalties, and the revenue collected is allocated to the IMO Net-Zero Fund. A two-tiered tariff system is proposed to encourage early adoption of green fuels, with higher costs for lower performing ships. The fund is intended to finance research and development by supporting the deployment of new technologies and helping developing countries manage the impact of this transition.

Alternative Fuels

Alternative Fuels, including green ammonia and methanol, hydrogen, and advanced biofuels are currently being explored to achieve long-term sustainable development goals. The transition to zero-emission vessels requires the replacement of conventional fuels with Scalable Zero-Emission Fuels (SZEFs).

Green Ammonia and Methanol: Green Ammonia (NH3) has emerged as a frontrunner for deep-sea shipping. Decarbonized and capable of being produced from green hydrogen and nitrogen, it has the potential to achieve zero lifecycle CO2 emissions. Its main advantages include established global energy storage infrastructure and the ability to be liquified at moderate temperatures. However, green ammonia presents several challenges, including its toxicity, lower energy density than heavy fuel oils, and the need for specialized engines. Conversely, green methanol (CH3OH) has taken an early lead in commercial adoption due to its ability to remain liquid at ambient pressure, making it easier to handle than ammonia or hydrogen. Methanol can be produced from biomass or as e-methanol (using green hydrogen and captured CO2), providing significant lifecycle GHG reductions, although its lower energy density requires larger fuel storage tanks, approximately two and a half times more volume than conventional fuel.

Hydrogen and Biofuels: Green hydrogen, produced by electrolysis using renewable electricity, is the cleanest option because its combustion produces only water. However, the use of hydrogen on deep-sea vessels faces insurmountable challenges. The main reasons are its incredibly low energy density per unit volume and the requirement for extreme storage conditions, such as cryogenic temperatures at -253 degrees C (20 K). While hydrogen fuel cells are practical for smaller, short-sea vessels, for deep-sea shipping they are more often used as a feedstock for producing ammonia or methanol. Biofuels, derived from organic waste or biomass, offers immediate “drop-in” compatibility with existing engines and infrastructure, allowing for significant emission reductions without the need for large-scale upgrades. However, their widespread deployment is limited by factors such as feedstock ability, concerns about sustainable land use, and competition for raw materials with other sectors, particularly aviation.

Bridge Fuels and Infrastructure Challenges: Liquified Natural Gas (LNG) is currently used as a transitional “bridge fuel,” reducing carbon dioxide emissions by 20-25% compared to Heavy Fuel Oils and decreasing sulfur oxides (SO_x) emissions. Despite this, methane slip – the release of unburned methane during combustion – can significantly reduce its climate benefits, as methane is a potent greenhouse gas. The transition to alternative fuel is hampered by a global shortage of bunkering (refueling) infrastructure (storage tanks, pipelines, jetties, and fueling equipment), with only a few ports currently prepared for methanol or ammonia bunkering. Furthermore, the inflated cost of clean fuels requires regulatory support to achieve economic parity with conventional fuels.

Improving vessel performance relies on various energy-saving technologies, such as propeller upgrades, hull cleaning, air lubrication systems, and advanced hull coatings, which reduces fuel consumption by up to 10%. Operational refinements, such as speed optimization (slow steaming), route planning, and efficient (“just in time”) port entry management, can also contribute to improved efficiency, reducing CO2 emissions by as much as 30%.

Infrastructure optimization focuses heavily on transforming ports into green energy hubs to support alternative fuels and shore-side power. This is a significant challenge, as the existing infrastructure is primarily designed for fossil fuels. The adoption of “cold ironing” – allowing ships to plug into the local electricity grid while at berth – significantly reduces emissions from auxiliary engines. Ports must develop bunkering infrastructure for low and zero-carbon fuels to support new, dual-fuel vessels.

Conclusion

The decarbonization of maritime shipping from conceptual ambition to mandatory action, driven by the International Maritime Organization’s 2023 strategy, which targets net-zero emissions by or around 2050, with critical checkpoints in 2030 and 2040. While regulatory frameworks such as the Net-Zero Framework create pressure for immediate, actionable changes, the industry faces significant, interconnected challenges. The success of the Net-Zero framework hinges on finalizing the implementation guidelines, including the adoption of fuel pathway certification and the exact mechanisms of the Net-Zero fund. While discussions continue, the general consensus remains that a global, mandatory, market-based measure is essential for the shipping industry to achieve its 2050 targets. Adoption of alternative fuels (including methanol, hydrogen, ammonia, biofuels, and LNG) is accelerating alongside operational efficiencies such as wind-assisted propulsion and air lubrication. However, the inflated costs of green fuels, a lack of global bunkering infrastructure, and the “chicken-and-egg” dilemma between fuel producers and vessel owners is hindering this transition. With the European Union implementing stricter emissions regulations starting in 2026, such as the Fuel EU Maritime initiative, the pressure for early adoption of alternative fuels is accelerating. Furthermore, given the long lifespan of existing vessels (15 to 40 years), rapid fleet modernization is necessary to avoid locking in high-carbon infrastructure. Looking towards 2030 and beyond, a fragmented, multi-fuel pathway is envisioned, heavily reliant on the adoption of global economic measures (such as the greenhouse gas levy) to bridge the cost gap. Ultimately, the success will depend on scaling up green energy supplies, upgrading port infrastructure, and fostering collaboration across the entire supply chain to ensure a just transition.

References

Blank Rome LLP. (2025, August 12). IMO Net-Zero Shipping Framework: A Crossroads for Global Shipping Regulations. https://www.zerocarbonshipping.com/imo-net-zero-framework

DNV. (2025, October 10). Net-zero shipping: Key findings from the latest Maritime Forecast.

www.dnv.com

Environmental Defense Fund Europe. (2025, January 22). Scalable green maritime fuels can help hit net zero by 2050. https://www.weforum.org/stories/2025/01/scalable-maritime-green-fuels/

Global Maritime Forum. (2026, April 20). A guide to the IMO's Net-Zero Framework. https://globalmaritimeforum.org/news/a-guide-to-the-imos-net-zero-framework

International Maritime Organization (IMO). (2025, April 11). IMO approves net-zero regulations for global shipping. https://www.imo.org/en/mediacentre/pressbriefings/pages/imo-approves-netzero-regulations.aspx

Laskar, I. I., & Giang, D. (2025, April 9). Expert Assessments of Maritime Shipping Decarbonization Pathways by 2030 and 2050. Wiley Online Library. https://agupubs.onlinelibrary.wiley.com/doi/full/10.1029/2024EF005255

MDPI. (2025, December 18). Sustainable Maritime Decarbonization: A Review of Hydrogen and Ammonia. https://www.mdpi.com/2071-1050/17/24/11364

Pole Star Global. (2025, May 1). Green Shipping Corridors Hub: Challenges and New Solutions. https://www.polestarglobal.com/resources/green-shipping-corridors/

Ports & Harbors. (2026, January 9). IMO 2026 Decarbonization Targets Drive Accelerated Green Corridor Investments. https://ports.marinelink.com/ports/port/cairo/news/imo-2026-emissions-targets-drive-green-corridor-investments

U.S. Department of Energy (DOE). (2025, January 22). DOE’s Action Plan for Maritime Energy and Emissions Innovation. https://stillwaterassociates.com/does-action-plan-for-maritime-energy-and-emissions-innovation-implications-for-biofuels/

Clear Seas. (2022). Decarbonizing Marine Shipping: Clean fuels for a greener future?

DNV. (2025). Net-zero shipping: Key findings from the latest Maritime Forecast.

Frontiers. (2022). The use of alternative fuels for maritime decarbonization.

Global Maritime Forum. (2025). Zero-emission shipping fuels: A guide to methanol and ammonia.

Leadvent Group. (2026). Sustainable Marine Fuels Challenges | Green Shipping Gap 2026.

MDPI. (2022). Estimates of the Decarbonization Potential of Alternative Fuels.

MDPI. (2025). Decarbonization Potential of Alternative Fuels in Container Shipping.

MDPI. (2026). Alternative Maritime Fuels for Net-Zero Shipping: A Comprehensive Operational, Techno.

Port of Seattle. (2022). Maritime Alternative Fuels 101.

US Ethanol Producer. (2026). US biofuels and alternative fuels: Charting the course for global marine decarbonization.

LOTUS Containers. (2025, September 16). Strategies for decarbonizing in shipping industry. https://www.lotus-containers.com/en/decarbonizing-in-shipping/

MDPI. (2025, March 26). Decarbonization of shipping and progressing towards reducing greenhouse gas…

Nautilus Shipping. (2025, December 4). Upcoming sustainability rules & regulations in 2026 for the maritime industry. https://www.nautilusshipping.com/news-and-insights/upcoming-sustainability-rules-regulations-in-2026-for-the-maritime-industry

Pacific Northwest National Laboratory (PNNL). (n.d.). Maritime decarbonization. https://www.pnnl.gov/explainer-articles/maritime-decarbonization

RMI. (n.d.). Maritime decarbonization. https://rmi.org/maritime-decarbonization/

ScienceDirect. (2025). Shipping sector decarbonisation measures: A review. https://www.sciencedirect.com/science/article/pii/S2213138825003807

World Resources Institute (WRI). (2023, October 25). How can the shipping industry reach net-zero emissions? https://www.wri.org/insights/how-to-decarbonize-international-shipping

World Ports Directory. (2026, January 5). IMO's tighter decarbonization mandates take effect in 2026. https://ports.marinelink.com/ports/port/naknek/news/imos-tighter-decarbonization-mandates-take-effect-in-2026

Center on Global Energy Policy at Columbia University SIPA. (2025, October 14). Practical gaps in the international maritime organization’s net-zero framework.

https://www.energypolicy.columbia.edu/publications/practical-gaps-in-the-international-maritime-organizations-net-zero-framework/

Environmental Defense Fund. (2025, October 17). IMO postpones adoption of net zero framework, delaying global action to decarbonize shipping. www.edf.org

Global Maritime Forum. (2026, February 19). Uncertainty at the IMO: Three scenarios and their consequences for shipping's transition. https://globalmaritimeforum.org/insight/three-scenarios-and-their-consequences-for-shippings-transition/

International Maritime Organization. (n.d.). IMO's work to cut GHG emissions from ships.

https://www.imo.org/en/mediacentre/hottopics/pages/cutting-ghg-emissions.aspx

International Maritime Organization. (2023). 2023 IMO Strategy on Reduction of GHG Emissions from Ships. https://www.imo.org/en/OurWork/Environment/Pages/2023-IMO-Strategy-on-Reduction-of-GHG-Emissions-from-Ships.aspx

Morgan, Lewis & Bockius LLP. (2025). What the Delay of the IMO’s Net-Zero Framework Means for Maritime Decarbonization. https://www.morganlewis.com/-/media/files/publication/outside-publication/article/2025/what-the-delay-of-the-imos-net-zero-framework-means-for-maritime-decarbonization-reuters.pdf

Sustainable-Ships.org. (2025). Upcoming Sustainability Rules & Regulations in 2026 for the Maritime Industry. https://www.nautilusshipping.com/news-and-insights/upcoming-sustainability-rules-regulations-in-2026-for-the-maritime-industry

UNCTAD. (2023). Review of Maritime Transport 2023 - Chapter 3: Decarbonizing shipping. https://unctad.org/system/files/official-document/rmt2023ch3_en.pdf

World Economic Forum. (2025). Decarbonizing the maritime industry with emissions tracking. https://www.weforum.org/stories/2025/05/decarbonize-maritime-industry-real-time-emissions-tracking/

This report is created in part with assistance from Google Gemini for drafting content related to the introduction, the IMO Net Zero Framework, alternative fuels, the improvement of port operations, and the conclusion section. This content was reviewed and edited by the author.